HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

FACULTY OF ELECTRICAL & ELECTRONICS ENGINEERING

DEPARTMENT OF CONTROL ENGINEERING & AUTOMATION

GRADUATION THESIS

APPLICATION OF IOT FOR MONITORING AND MANAGING HOUSE

HO CHI MINH CITY, JULY 2020

HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

FACULTY OF ELECTRICAL & ELECTRONICS ENGINEERING

DEPARTMENT OF CONTROL ENGINEERING & AUTOMATION

GRADUATION THESIS

APPLICATION OF IOT FOR MONITORING AND MANAGING HOUSE

SUPERVISOR : PhD NGUYEN TRONG TAI

STUDENT : PHAM MINH THUAN – 1613435

HO CHI MINH CITY, JULY 2020

HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

Electrical & Electronic Engineering

SOCIALIST REPUBLIC OF VIETNAM

Independence – Freedom – Happiness

Control Engineering & Automation

GRADUATION EVALUATION OF SUPERVISOR

Name of Thesis:

IOT APPLICATIONS IN THE CONSTRUCTION OF EQUIPMENTS MONITORING AND MANAGEMENT SYSTEM IN HOME

Student:							Supervisor:

Pham Minh Thuan			1613435		PhD Nguyen Trong Tai

Eluvation:

- Thesis book:

Pages:			____________		Chapters:	____________

Tables:			____________		Picture:	____________

References:   		____________		Product(s):	____________

Remarks about the presentation of  the thesis book:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Thesis evaluation:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Potential applications:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Working attitude of student:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

Overall evaluation:

Score of student:

Pham Minh Thuan:         …………/10

Supervisor

(Sign and full name)

HO CHI MINH CITY UNIVERSITY OF TECHNOLOGY

Electrical & Electronic Engineering

SOCIALIST REPUBLIC OF VIETNAM

Independence – Freedom – Happiness

Control Engineering & Automation

GRADUATION EVALUATION OF SUPERVISOR

Name of Thesis:

APPLICATION OF IOT FOR MONITORING AND MANAGING HOUSE

Student:							Judges:

Pham Minh Thuan			1613435

Eluvation:

- Thesis book:

Pages:			____________		Chapters:	____________

Tables:			____________		Picture:	____________

References:   		____________		Product(s):	____________

Remarks about the presentation of  the thesis book:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Thesis evaluation:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Potential applications:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

- Working attitude of student:

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

________________________________________________________________________________________________

Overall evaluation:

Score of student:

Pham Minh Thuan:         …………/10

Judge

(Sign and full name)

ACKNOWLEDGEMENT

To complete my thesis, I would like to  express my respect and gratitude to PhD Nguyen Trong Tai. My supervisor give me an great environment  to complete my thesis. The instruction of  PhD Nguyen trong Tai helped  me to go right way and have a successful solution.

Contents

TÓM TẮT LUẬN VĂN	2

CHƯƠNG 1: GIỚI THIỆU ĐỀ TÀI	3

1.1 Tính cấp thiết của Luận văn	3

1.2 Mục tiêu của Luận văn	4

1.3 Đối tượng và phạm vi nghiên cứu	5

1.4 Nhiệm vụ và nội dung của Luận văn	5

CHƯƠNG 2: GIỚI THIỆU TỔNG QUAN VỀ HEXAPOD	7

2.1.  Tổng quan về thiết kế và điều khiển tay máy song song Hexapod theo mô hình Stewart – Gough Platform	7

2.1.1 Giới thiệu sơ lược về tay máy song song	7

2.1.2 Tình hình nghiên cứu về tay máy song song kiểu Stewart – Platform	8

2.2 Phân loại Hexapod	14

2.3 Mô hình hóa Hexapod	18

2.3.1 Lựa chọn mô hình thực hiện	18

2.3.2 Phương trình động học mô tả hệ thống	20

2.4 Khảo sát khả năng hoạt động của mô hình đã xây dựng	27

2.4.1 Yêu cầu của mô hình	27

2.4.2 Thông số đề xuất	28

2.4.3 Khảo sát khả năng hoạt động của mô hình bằng Matlab	28

CHƯƠNG 3: THIẾT KẾ, THI CÔNG MÔ HÌNH HEXAPOD	47

3.1 Thiết kế mô hình bằng SolidWorks	47

3.2 Bản vẽ thiết kế	47

CHƯƠNG 4: BỘ ĐIỀU KHIỂN, PHẦN MỀM VÀ GIẢI THUẬT  ĐIỀU KHIỂN MÔ HÌNH	61

4.1 Bộ điều khiển Hexapod và các thiết bị chấp hành	61

4.1.1 Chip điều khiển STM32F407VG	61

4.1.2 Chip điều khiển Max II EMP570GT100C4	61

4.1.3 Driver PID điều khiển động cơ Servo	62

4.1.4 Thiết kế mạch điều khiển	63

4.1.5 Động cơ điều khiển	69

4.2 Xây dựng phần mềm điều khiển	70

4.2.1 Xây dựng phần mềm điều khiển trên máy tính bằng phần mềm Visual Studio V15	70

4.2.2 Xây dựng phần mềm điều khiển bằng màn hình Weintek 6071iP	73

4.3 Sơ đồ hệ thống kết nối và giải thuật điều khiển	76

4.3.1 Sơ đồ hệ thống kết nối	76

4.3.2 Giải thuật điều khiển	78

CHƯƠNG 5: THỬ NGHIỆM MÔ HÌNH	80

5.1 Công cụ sử dụng để thử nghiệm mô hình	80

5.2 Một số kết quả kiểm tra mô hình theo các trục	81

5.2.1 Kiểm tra độ chính xác của mô hình theo trục z	81

5.2.2 Kiểm tra độ chính xác của mô hình theo trục x, y	83

5.2.3 Chạy mô hình theo các quỹ đạo định sẵn	85

CHƯƠNG 6: KẾT LUẬN VÀ HƯỚNG PHÁT TRIỂN	86

6.1 Kết luận	86

6.2 Hướng phát triển của đề tài	86

TÀI LIỆU THAM KHẢO	88

List of Tables

Bảng 2.1 So sánh các dòng Hexapod có khả năng đáp ứng vị trí độ chính xác cao	14

Bảng 2.2 So sánh các dòng Hexapod có khả năng đáp ứng chuyển động	15

Bảng 2.3 So sánh các dòng Hexapod theo cơ cấu và cách thức truyền động	16

Bảng 2.4 So sánh ưu nhược điểm của các mô hình	17

Bảng 2.5 Chi tiết các bộ phận chính của mô hình	19

Bảng 2.6 Thông số mô hình Hexarotor	28

Bảng 2.7 Một số thông số chính của mô hình Hexapod	28

Bảng 2.8 Khảo sát ảnh hưởng từng thông số  đến vùng hoạt động của mô hình	33

Bảng 2.9 Khảo sát mô hình cho 4 thông số cùng chạy	37

Bảng 2.10 Khảo sát vị trí của tấm chuyển động với một trục x, y hoặc z cố định	39

Bảng 2.11 Khảo sát vị trí của tấm chuyển động khi cho một góc Euler cố định	43

Bảng 5.1 Một số thông số mô hình	80

Bảng 5.2 Bảng thông số cảm biến đo khoảng cách HG-C1400	81

Bảng 5.3 Kết quả thử kiểm nghiệm độ chính xác mô hình theo trục z	82

List of Figures

Hình 1.1 Tàu sân bay Washington, ảnh chụp tháng 6, 2011	3

Hình 2.1 Mô hình tay máy song song kiểu Stewart – Gough Platform	7

Hình 2.2 Tay máy song song của hãng Dunlop Rubber	9

Hình 2.3 Thiết bị mô phỏng bay của tiến sĩ Stewart	10

Hình 2.4 Delta Robot	10

Hình 2.5 Thiết bị gia công Hexapod – dự án của NIST	11

Hình 2.6 Mô hình Hexapod PR6-02 trên thực tế	12

Hình 2.7 Ứng dụng trong gia công cơ khí	13

Hình 2.8 Ứng dụng trong Y học	13

Hình 2.9 Ứng dụng trong mô phỏng	13

Hình 2.10 Ứng dụng trong thiên văn	13

Hình 2.11 Ứng dụng trong trắc địa	13

Hình 2.12 Mô hình mẫu thiết kế Hexapod	19

Hình 2.13 Hệ tọa độ quy chiếu	21

Hình 2.14 Các hệ quy chiếu trong mô hình tay máy Hexapod	22

Hình 2.15 Minh họa tọa độ của chân và Servo	23

Hình 2.16 Hình chiếu trên mặt phẳng song song với Base	24

Hình 2.17 Phần mềm khảo sát được xây dựng bằng Matlab – guide.	29

Hình 2.18 Vẽ mô hình trong không gian khi tấm payload ở vị trí “Home”	30

Hình 2.19 Tỷ lệ đạt được vùng hoạt động khi Platform Joint Angle thay đổi	33

Hình 2.20 Tỷ lệ đạt được vùng hoạt động khi Base Joint Angle thay đổi	34

Hình 2.21 Tỷ lệ đạt được vùng hoạt động khi Platform Radius thay đổi	34

Hình 2.22 Tỷ lệ đạt được vùng hoạt động khi Base Radius thay đổi	35

Hình 2.23 Tỷ lệ đạt được vùng hoạt động khi Servo Arm Length thay đổi	35

Hình 2.24 Tỷ lệ đạt được vùng hoạt động khi Connecting Arm Length thay đổi	36

Hình 2.25 Tỷ lệ đạt được vùng hoạt động khi Default Z Height thay đổi	36

Hình 2.26 Tỷ lệ đạt được vùng hoạt động khi cho 7 thông số mô hình thay đổi	38

Hình 3.1 Mặt trước của mô hình	48

Hình 3.2 Mô hình nhìn từ trên xuống	48

Hình 3.3 Bản vẽ chốt bắt ổ bi	49

Hình 3.4 Bản vẽ trục gài ổ bi giữa cánh tay chống và cánh tay quay	50

Hình 3.5 Bản vẽ trục bắt ổ bi giữa tay chống và bảng bắt khớp	51

Hình 3.6 Bản vẽ bảng bắt động cơ	52

Hình 3.7 Bản vẽ tấm đế dưới (Base)	53

Hình 3.8 Bản vẽ thanh chống chân kết nối với tấm chuyển động (payload)	54

Hình 3.9 Bản vẽ tấm chuyển động (payload)	55

Hình 3.10 Bản vẽ trục của cánh tay chống	56

Hình 3.11 Bản vẽ cánh tay quay	57

Hình 3.12 Bản vẽ lắp của mô hình Hexapod hoàn chỉnh	58

Hình 3.13 Ảnh chụp mặt trước của mô hình	59

Hình 3.14 Ảnh chụp mặt dưới mô hình từ dưới lên	60

Hình 4.1 Chip STM32F407VG của ST	61

Hình 4.2 Chip Max II EMP570GT100C4 Altera	62

Hình 4.3 Mạch điều khiển động cơ DC Servo PID 200W MSD-E21V1	62

Hình 4.4 Khối nguồn	64

Hình 4.5 Khối mạch STM và CPLD	64

Hình 4.6 Khối ENCODER	65

Hình 4.7 Khối ADC	65

Hình 4.8 Một số khối giao tiếp	66

Hình 4.9 khối PWM	66

Hình 4.10 Khối Input	67

Hình 4.11 Khối Output	67

Hình 4.12 Ảnh 3D mặt trước của Main Board	68

Hình 4.13 Ảnh 3D mặt sau của Main Board	68

Hình 4.14 Ảnh Main board thực tế	69

Hình 4.15 Động cơ DC Servo Harmonic được sử dụng	70

Hình 4.16 Phần giao diện thực hiện chức năng chạy mô hình.	72

Hình 4.17 Phần kết nối cổng COM giữa máy tính với mô hình	72

Hình 4.18 Vùng cài đặt thông số mô hình	73

Hình 4.19 Vùng điều khiển mô hình	73

Hình 4.20 Màn hình chính điều khiển chương trình.	74

Hình 4.21 Phần cài đặt thông số các động cơ cho mô hình	74

Hình 4.22 Phần điều khiển trực tiếp mô hình	75

Hình 4.23 Phần cài đặt các thông số mô hình	75

Hình 4.24 Mô hình kết nối hệ thống	77

Hình 4.25 Giải thuật điều khiển mô hình	78

Hình 4.26 Giải thuật nội suy đường thẳng theo phương pháp xung chuẩn	79

Hình 5.1 Cảm biến HG-C1400	80

Hình 5.2 Bố trí thử nghiệm độ chính xác theo trục z của mô hình	81

List of Keywords

| Keyword | Full | Meaning |
| --- | --- | --- |
| ADC | Analog to Digital Converter | Bộ chuyển đổi tín hiệu tương tự sang tín hiệu số |
| ASME | American Society of Mechanical Engineers | Hội kỹ sư cơ khí Mỹ |
| ARM | Advanced RISC Machine | Cấu trúc vi xử lý kiểu RISC |
| CAN | Controller Area Network | Chuẩn giao tiếp giữa vi điều khiển và ngoại vi không cần host computer |
| CNC | Computer Numerical Control | Hệ thống máy gia công cơ khí điều khiển bằng máy tính |
| CPLD | Complex Programmable Logic Device | Vi mạch khả lập trình được xây dựng trên nền EEPROM |
| CPU | Central Processing Unit | Bộ xử lý trung tâm |
| DAC | Digital to Analog Converter | Bộ chuyển đổi tín hiệu số sang tín hiệu tương tự |
| DC | Direct Current | Dòng điện một chiều |
| DMA | Direct Momory Access | Phương pháp cho phép thiết bị vào / ra gửi và nhận dữ liệu trực tiếp không cần thông qua CPU |
| FSMC | Flexible Static Memory Controller | Bộ điều khiển giao tiếp bộ nhớ ngoại vi |
| GUI | Graphical User Interface | Giao diện đồ họa người dùng |
| HMI | Human – Machine Interface | Giao diện tương tác người – máy |
| I2C | Inter – Intergrated Circuit | Chuẩn giao tiếp nối tiếp 2 dây giữa vi điều khiển và ngoại vi |
| IC | Integrated circuit | Vi mạch tích hợp |
| IDE | Intergrated Development Environment | Môi trường phát triển / lập trình tích hợp |
| IEEE | Institute of Electrical and Electronics Engineers | Viện Kỹ sư Điện và Điện tử |
| IFAC | International Federation of Automatic Control | Liên đoàn quốc tế về điều khiển tự động |
| IFToMM | International Federation for the Promotion of Mechanism and Machine Science | Liên đoàn quốc tế về Phát triển khoa học máy và Cơ cấu |
| IMU | Inertial Measurement Unit | Cảm biến góc quay |
| Matlab | Matrix Laboratory | Phần mềm cung cấp môi trường tính toán số và lập trình |
| MCU | Micro Controller Unit | Vi điều khiển |
| NIST | National Institute of Standards and Technology | Viện tiêu chuẩn và công nghệ quốc gia (Mỹ) |
| PC | Personal Computer | Máy tính cá nhân |
| PID | Proportional Integral Derivative | Bộ điều khiển vi tích phân tỷ lệ – Bộ điều khiển PID |
| PWM | Pulse Width Modulation | Điều chế độ rộng xung |
| SPI | Serial Peripheral Interface | Giao tiếp ngoại vi nối tiếp (SPI) |
| SRAM | Static Random Access Memory | Bộ nhớ truy cập ngẫu nhiên tĩnh |
| UART | Universal Asynchronous Receiver – Transmitter | Chuẩn truyền thông nối tiếp bất đồng bộ |
| USB | Universal Serial Bus | Chuẩn kết nối nối tiếp có dây trong máy tính |
| USART | Universal Synchronous Asynchronous Receiver – Transmitter | Chuẩn truyền thông nối tiếp bất đồng bộ / đồng bộ |

ACKNOWLEDGEMENT

IoT is the field, which is developing very fast. We have many project, which is application of IoT such as : Smart-home, green-agriculture, IoT in Industrial,… But a lot of project just be build for 1 invidual, it mean it just can be operate in your local area. So when we want to connect with other project, we need to write the driver to interface, or event we need to build other application again.

In the world, we have many application , cloud support IoT, which can help us resolve this problem, such as : Blynk, Google cloud IoT core, Azure, AWS IoT,… they are extremely great, but we need to pay money to use it.

In this thesis, I endeavor to create a system, which include : 1 IoT-core (websever, database) , 1 template for micro-controller esp32 to communicate with IoT-core, 1 webapp to users can use to manage your house. My system support muilti-user, it mean many people can create your account and start to use it now. With HTTP protocol, not only esp32 serials but also a smart-phone, PC, laptop, stm32-serials, orther microcontroller, microprocessing... which can become a device and allow user to  manage them. With using free platform, free language, free framwork, I will give for people a free application to use, develop. It allow developer can expanse feature, do not build again, create mobile-app with using API from Server, build many platfrom for other microprocessing/microcontroller, with users, it is easy to use or fix.

# Chapter 1:

# INTRODUCTION

  - Thesis overview

IoT or Internet of things describes a system, which include many interrelated computing devices, every “thing” has a ID and they can share data together or communicate with human. IoT is not limited by network , protocols or technology.

Applications of IoT appeared in home, Industrial Automation, Agriculture and so on. The common project we can mention : Smart-Home, Home Automation, Green Agriculture, Health Monitoring system, Smart Parking System.

Because I do not want the low-level belong to hight-level  so much, my application have to allow users to choose option in a low-level and then config it on a hight-level to use, or just config whatever you want, then you can program device with function you have choosen. So, I decided to research some pattern system to learn how to do.

The figure below is  structure of Google Cloud IoT Core(1),which give me a ideal :

Figure 1.1 Diagram of Google Cloud IoT Core

That figure describe  how to Google Cloud IoT Core operation,  it will be a processing center and controller center, it also look like an environment where devices, provisoner, clouds become a block, and they can communicate with one another.

Because I feel this architecture of Google is so cool, in addition, I am expressed about mobile-app “Blynk”(2). All of them make me really want to create an Application, which can support GUI like “Blynk” but it is on the web browser ( not only on the mobile phone but also PC(3),  tablet,…) , and my application have a enduring struct  to support muilti-user, muilti-device.

Finally, I decide make a lightweight, powerful system, it have a API(4), have view engine, flow this diagram below:

Figure 1.2 Diagram of system in thesis

IoT Core

This element is headquater of system, it like a “Big bridge” help other element can operate synchronously, smoothly. IoT core of my system is a websever, it is based on MVC(5) architectual.

User

This element represent for device like smartphone, PC,… or anything that is supported REST(6) and HTTP/HTTPS(7).

Device

This element is microprocessors, microcontrollers, which is supported   HTTP, TCP/IP. Developer can program them for your purposes and users just use it with pattern platform.

Database

This element is used to store data, manage them flexibl. All data of user , data of command will be save here.

Sensor

This element represent for sensor , devices,…. Which collect data from environment, help the users in monitoring.

Equipment

This element represent for machines, devices which the users can control and manage.

1.2 Objective

The objective of the thesis is to build a system which can be used to monitor and manage the house, it is able to extend for many other IoT(8) applicaion. In addition, hardware of device and system is not belong each other, it mean developers can create PCB and then program firmware with peripheral they want, after that , they just config on server and use it.

In order to gain this, the thesis needs to follow some tasks-core:

- Building a websever with MVC.

- Building a GUI(9) to users can communicate easily.

- Building template(10) for devices to interface with websever.

- Creating a database with simple form and lightweight storage.

- Building driver which support for communicating between device and sensors,equipments.

1.3 Thesis structure

I divide thesis into 6 chapter , belong to task and element of system:

Chapter 1 : Introdutrion : I will present overview about IoT , some Application about IoT , reason I want to create this system, mention issues when I begin and prerequisite tasks I need to gain in this thesis.

Chapter 2: IoT core : I will go to details about it, the reason I choose websever, how to build it with MVC architectual,  how to communicate with device, how to build webapp with view engine.

Chapter 3: Database : I will introduce about MongoDB(11) , reason I choose it and which driver I chose to communicate with MongoDB.

Chapter 4: Device : I will present overview about ESP32 chip, how I use esp-idf to build platform for esp32 microcontroller, how to using JSON and Protobuf to communicate with IoT Core.

Chapter 5: User : I will introduce abount REST(12) and RESTful API(12) , how users can communicate with IoT core through GUI of webapp and manage your device through IoT core.

Chapter 6: Summary : I will introduce about results, limitations and development oritenation in the future of thesis.

# Chapter 2:

# IOT CORE

2.1.  Structure of IoT core.

The my IoT Core is a websever, it is build from:  JavaScript, HTML, CSS, PUG, NPM . JavaScript is a language for client, it mean it just run on browser like Firefox, CocCoc, Chomium, Edge,...But with the platform Nodejs, which have powerful  engine V8, I can build my websever on it. With support from NPM , an open source, I can build sever in the great way.

Javascript do not have a “top function”, we will not have function like “int main()” in file named “main.js”. In theory , I can add anything in 1 file, but it will look terrible, and will be hard to fix bug. Those files need to be sorted into folder tree. But how? and what is standard ?

2.1.1.MVC architectual pattern.

“The Model-View-Controller (MVC) is an architectural pattern that separates an application into three main logical components: the model, the view, and the controller. Each of these components are built to handle specific development aspects of an application. MVC is one of the most frequently used industry-standard web development framework to create scalable and extensible projects.”( https://www.tutorialspoint.com/mvc_framework/index.htm)

Model

“The Model component corresponds to all the data-related logic that the user works with. This can represent either the data that is being transferred between the View and Controller components or any other business logic-related data. For example, a Customer object will retrieve the customer information from the database, manipulate it and update it data back to the database or use it to render data”.

View

“The View component is used for all the UI logic of the application. For example, the Customer view will include all the UI components such as text boxes, dropdowns, etc. that the final user interacts with”.

Controller

“Controllers act as an interface between Model and View components to process all the business logic and incoming requests, manipulate data using the Model component and interact with the Views to render the final output. For example, the Customer controller will handle all the interactions and inputs from the Customer View and update the database using the Customer Model. The same controller will be used to view the Customer data”.

2.1.2 Inside my IoT Core.

I had spent a long time to understand MVC structure, and inherited their ideal. After all that, I represent a structure of my IoT Core :

Figure 2.1 Diagram of IoT Core in thesis

With this structure, I start to actualize it. First of all, I organize a directory tree like below :

Figure 2.2 directory tree in the sever

I will explain folders and file one by one to make it easy to understand compatible things with MVC

- Files in the top folder:

Figure 2.3

- app.js :  it is the most important file,  it will require package, config parameter or method of sever, setup database , declare target for URL  , setup view engine and start sever.

- Package.json : it is is default file, which contain some setting on sever , like package used in the sever, name of main file, scripts to run.

- Package-lock.json : it is default file , which help reduce error when we add 2 version of 1 pakage.

- .gitignore : because I use github, I will add some file or folder which do not update when I fix my sever.

- Procfile : to declare the script to run sever, my sever is located in Heroku, which give me a resource , a domain name to use, so I need this file to Heroku know how to run.

- start.js : this is a file  to manager the platform dasdboard, I use platform patern for view engine to make a GUI.

- .vscode

Figure 2.4

this folder is generated when I use Visual Studio Code, it contain some settings of developers, but with my sever, it just be a empty file.

- routers

Figure 2.5

The sever have a URL to browser can request to it , every path is a target of main URL “https://iot-server-365.herokuapp.com/” . Every file routers manage 1 or some path.

- indexs.js : this manages router ‘/’, it is in roof of path. Because I think go to login page when user access webapp is not interesting.

- logins.js : this file will check information of users, if it is valid, users will receive a cookie to access your resourch, until they log out.  It manages router ‘/login’.

- registers.js : this file will check information of registers, if they are new users, sever will create a new instance and store in database. It manages router ‘/register’.

- storages.js : this file manages router ‘/storage’, it is a gate to communicate with devices, where data will be post and command will be get.

- test.js : this file is test router , it helpful for creator to test a new add on feature.

- users.js : this file manages 5 routers :

- ‘/user’  : this router manages main page of user, this page is a dashboard, it give user many option and show some chart.

- ‘/user/update’ : this router manages update page, user can change your ID device , or change your password

- ‘/user/clone’ : this router manages clone page, this page is allow users clone your data they receive from devices with Excel, PDF, CSV format , or just print if users are connecting printer.

- ‘/user/logout’ :  this router reset cookie of users and redirect them to login again, users just press ‘logout’ button.

- ‘/user/delete’ : this router will delete account of user, they will become register again, user should consider carefully.

- config.js : this file manages 7 routers:

- ‘/user/config’ : this router manages config page, this page allow users to add , remove device esp32 .

- ‘/user/config/sensors’ and ‘/user/config/equipments’: these routers manage sensor page and equipment page, these pages allow users to see your device’s record.

- ‘/user/config/sensors/search’ and ‘/user/config/equipments/search’: these routers will receive name of device you fine and go to its page, I think it is useful when you have many devices.

- ‘/user/config/sensors/:id’ and ‘/user/config/equipments/:id’ : these routers manage a page which contains parameter of device esp32.

- forgetpass.js: this file is a method to help users when they forget your password and can not login, it include 2 routers :

- ‘/forgetpass’: users will send to this router their email, and it will send a authentication message to their email.

- ‘/forgetpass/verify’: this router will check your number of authentication message and check users’s email again, if their information is correct, router will send new password to their email, users can login and change other password or just use that password.

- guis.js: this file manages routers which support and manage blocks. Block is a cluster which inlucdes 1 or many equipments, one block can contain equipments from other devices. File “guis.js” includes 4 routers:

- ‘/user/gui’: this router is manages gui page, this page allow users add or remove their block, block corresponds port in device, and it have to assign with 1 device.

- ‘/user/gui/blocks’: this router manages block page, it show a record of block which you added, it support search bar to fine block with its name. It receives API which is command from users and save to database.

- ‘/user/gui/blocks/search’: this router will receive name of block you need and then redirect to page of this block, it help full when you have many blocks.

- ‘user/gui/block/:id’: this router manages 1 router which match its id, it will render 1 page to users can control easily.

- displays.js : this file manages routers which support chart and show data from sensors. It includes 3 routers :

- ‘/user/display’: this router will render display page with 2 kind of chart : line chart and bar chart.

- ‘/user/display/getdata’: this router will help display page init data and send lastest data to display page to perform. It just support line chart and bar chart.

- ‘/user/display/datatable’: this router will support for table, it will send data when it receives correct request

- middlewares

s

Figure 2.6

Some router have to do many operation, every action in my sever have to check information of data first, then sever will decide to do it or no.

It contains 6 file for 6 validation purpose :

- auth.middleware.js: this file has 1 middleware, its name is ‘requireAuth”, it will check cookies of users, the cookies is just received when users login, so this module like a filter to prevent stranger and protect data of users.

- data.validate.js: this file has 1 middleware , its name is “checkID” , I want that only devices which have IDs of users and correct form  will be accepted and stored

- delete.validate.js: this file has 2 middleware:

- “checkFilled” : to ensure that users fill all necessary information.

- “checkAccount” : Deleting account means users will lost all data they have, I want to check information carefully, in case, the strangers can access to account of users, they can not delete it because stranger don’t know password.

- login.validate.js: this file is 2 middleware likes delete.validate.js file, but it render login page instead of delete page.

- register.validate.js: this file likes login.validate.js file, too. But, it add a function which is used to send email t verify ID for users.

- update.validate.js: this file like login.validate.js file, too. But it will render update page.

- controller

Figure 2.7

Controllers inlucde final actions of functions callback in processing request, dividing callback into stages will help to fix easily, increase perform, I create 5 file controller:

- data.controller.js: it just sends one response to devides which post data successfully.

- delete.controller.js : it will render delete page if cookies is correct, or redirect users to login page if deleting is successful.

- login.controller.js : it will render login page if we access login router, if we login successful, it will make a history login into database, and redirect users to user page.

- register.controller.js : it will redirect register to login page to access if regis successfully.

- update.controller.js: it will update new data of users on database, if users update ID , it will send email with new ID .

- models

To be store in the database, data need to arrange with type JSON(11), and some functions which is reused many time. Form and function in these file just are called and use in other file, do not change their struct.

- command.js : this is model of command to store in database, commands are used to control device :

-------------------------------------------------------------------

var mongoose = require('mongoose');

var commandSchema = new mongoose.Schema({

ID: String,

timestamp: String,

device: Number,

io: Number,

value: Number

},{ versionKey: false});

var Command = mongoose.model('Command', commandSchema, 'command');

---------------------------------------------------------------------------------------------

- data.js: data model is form to save from device to database.

---------------------------------------------------------------------------------------------

var mongoose = require('mongoose');

var dataSchema = new mongoose.Schema({

ID: String, /** unique Account */

device: Number, /** unique device, odd for post/display, even for get/gui */

datetime : String, /** for display/save realtime */

timestamp: Number, /** for caulator realtime */

form: [Object] /** data from device */

},{ versionKey: false});

var Data = mongoose.model('Data', dataSchema, 'data');

---------------------------------------------------------------------------------------------

- email.js: this is model to send authentication email, verify email. It can be reuse in other feature .

---------------------------------------------------------------------------------------------

module.exports.mailServer = nodemailer.createTransport({

service: 'gmail', //https://medium.com/@.jay/sending-email-using-express-js-with-nodemailer-in-heroku-71741f29463c

host: 'smtp.gmail.com',

port: 465,

secure: true,

auth: {

user: process.env.EMAIL,

pass: process.env.PASS_EMAIL

}

});

module.exports.form = {

from: "Server IoT core Te",

to: " ",

subject: " ",

text: " "

};

---------------------------------------------------------------------------------------------

- history.js: this model store state of users, login or logout.

---------------------------------------------------------------------------------------------

var mongoose = require('mongoose');

var historySchema = new mongoose.Schema({

timestamp: String,

email: String,

act : Number

},{ versionKey: false});

var History = mongoose.model('History', historySchema, 'history');

---------------------------------------------------------------------------------------------

- manageDev.js : this model is form to save state of equipments, data from sensors, save parameters of devices.

---------------------------------------------------------------------------------------------

var mongoose = require('mongoose');

var manageDevSchema = new mongoose.Schema({

ID: String, /** unique Account */

timestamp: Number,

dev : Number,

mask: String,

type : Number,

child : [Object]

},{ versionKey: false});

var ManageDev= mongoose.model('ManageDev', manageDevSchema, 'manageDev');

---------------------------------------------------------------------------------------------

- timestamp.js : this is model to log timestamp with information, it is used to determine time have bug, or time data processing of IoT core, time response device.

---------------------------------------------------------------------------------------------console.log('Timestamp: ', Date.now());

next();

---------------------------------------------------------------------------------------------

- user.js: this model is used to save information of users, ‘status’ announces some state of users, it can be expanse and useful in the future.

---------------------------------------------------------------------------------------------

var mongoose = require('mongoose');

var userSchema = new mongoose.Schema({

timestamp: Number,

email: String,

password: String,

status: Number

});

var User = mongoose.model('User', userSchema, 'user');

---------------------------------------------------------------------------------------------

- protobuf

Protobuf is a language-neutral of google , it a good way to make a API like JSON and XML. It will serialize a form data into an array, size of one element in this array is 1 byte.

I use JSON to make a form data which sent from device, but I use protobuf to make a form command which sent from IoT Core. There 2 reasons : C language does not have object concept, so analyzing a JSON string is so hard; the secondly, protobuf is so easy to deploy. I made 2 protobuf form :

- message.proto: This is form to send command to devide, because I control DAC, PWM, Digital and they are assigned to pin on device, I can send value of pin to control  it.

---------------------------------------------------------------------------------------------

message Sensor {

string ID=1;

uint32 device=2;

uint32 io =3;

float value=4;

}

message Sensors {

repeated Sensor sensors=1;

}

---------------------------------------------------------------------------------------------

- notify.proto: This form is used to send a message to device. Firstly, I create it to send a message to device and it will display on a screen or may be used to send message between users like Messanger. After thinking, I decided to do not do that, and it may be useful in future.

---------------------------------------------------------------------------------------------

message Notify  {

string ID=1;

uint32 device=2;

string header=3;

string content=4;

string comment=5;

}

message Notifies {

repeated Notify notifies=1;

}

-------------------------------------------------------------------

- Syntax to generate proto file to JavaScript file:

---------------------------------------------------------------------------------------------

protoc --js_out=import_style=commonjs,binary:. file.proto

-------------------------------------------------------------------

After generating this file, we need core runtime, which should be in a file called “google-protobuf.js”, it can be installed through npm “google-protobuf”.Finally, we can use output file nomally.

- Syntax to generate proto file to file.c and file.h in C:

---------------------------------------------------------------------------------------------

protoc-c --c_cout=. file.proto

---------------------------------------------------------------------------------------------

After generating this file, we will have 1 components with source file and header file.

- views

IoT core use a view engine named “pug”. Pug is an HTML preprocessor implemented in Node.js and it  help us render page from Pug to HTML. It supports methods, packages to make the pages of users look better. I use 1 Platform pattern, which design on github of David Miller. It is include : layouts and pages folder. My pages I create is based on pattern and layout of that resources.

- layouts:

There are features standards and some forms which are unchange, all of that will be render into layouts. I take it and change some feature which are usefull with my thesis.

- css.pug: it includes sources file CSS, they are pattern which is design to make pages look so great.

- icons.pug: this file add icon of webapp.

- meta.pug: it includes meta tags, kind of tag defines metadata  about an HTML document. Metadata is data (information) about data

- title.pug: this file help every page is assign title when sever render.

- sidenav.pug: it support format on the left of dashboard form.

- topnav.pug: it support format on the top of dashboard form.

- footer.pug:  it support format on the bottom of pages.

- scripts.pug: it include script tag to add pattern JavaScript files.

- authentication.pug: it is a pattern layout, which is used to make a login page or register page.

- dashboard.pug: it is pattern dashboard, I fix it to render for user page.

- error.pug: it is pattern layout for error page, when users access wrong URL, this layout will be rendered.

- pages:

These files are pattern pages, I build my pages with those form with fixing and adding JavaScript file to catch event.

- datatable.pug: this file form of one table in HTML, data table is support bt jQuery library.

- page-header.pug: this file support format on the top of page.

- 401.pug : it is a pattern layout, rendered with 401 error, 401 error occur when request of client has not been applied.

- 404.pug : it is a pattern layout, rendered with 404 error, 404 error occur when pages are not found.

- 500.pug : it is a pattern layout, rendered with 500 error, 500 error happen when something has gone wrong on sever.

Next, I will present my pages I made:

- index.pug : my home is rendered from this file, it is very simple, and It make my me, not using pattern layouts.

- login.pug : this file will be rendered to login page,  anyone wants to go to path ‘/user’ , they have to pass this page.

- register.pug : this file is rendered to register page, if users do not an account, they need to go to this page.

- forgetpass.pug : I make file to render forgetpass help page, users can go to this page to reset password.

- verify.pug : this file is rendered to verify page, this page help users verify you email to reset.

- user.pug : this file base on pattern dashboard layout, but I add some feature to make it useful on my thesis.

- update.pug : this file is rendered to update page, users can update new password or receive new ID.

- delete.pug : this file is rendered to delete page, users can delete your account in this page.

- tables.pug : this file base on pattern tables layout, it help to display data with tables

- charts.pug : this file base on pattern charts layout, I using chart.js package for it.

- config.pug : this file base on pattern login layout, just adding more input tags and more buttons tag.

- sensors.pug : this file is rendered to sensor page, it is base on pattern login layout.

- sens.pug : this file is rendered to sensi page, it is base on pattern login layout.

- equipments.pug : this file is rendered to equipment page, it is base on pattern login layout.

- equips.pug : this file is rendered to equipi page, it is base on pattern login layout.

- guis.pug : this file is rendered to equipi page, it is base on pattern login layout, it is quite the same of  “config.pug”.

- blocks.pug : this file is rendered to block page, it is base on pattern login layout.

- blocki.pug : this file is rendered to page, based on index of block , it is base on pattern login layout.

- public

It includes three folder : images, javascript, stylesheets. In order to have a good view, I need CSS files , it will increase UX . These folder represent three parts of one page : content, format and event.

- images

I put my images in here, “user.jpg” is used in the first page when user access sever, “icon.png” is icon on all pages of my webapp. Other pictrue is old, it may be used in the future.

- stylesheets

In the first time, I just want to make a sever API , but then , I think the GUI shoud be create to users can use easily. The pages need a good format, it help full to make users expressed.

CSS help HTML look so great, nobody wants a page with only basic tag. I have collected many CSS files on the internet and put ti here, but there are 3 important files :

- my_styles.css: this file includes format of pattern layouts I use, it have many good properties.

- style.css: this file is made by me, some properties I made and some properties I have coppied from other pages like: FaceBook, Google,Lazada.

- rangeSlither.css: this file support format properties to make a slider, which I use on control DAC and PWM of device.

- javascripts

Every page needs one or more JavaScript files to catch events and process them. This is JavaScript files for clients.

- tool.js: this file will get time of client and process and display on the top of home page, like a clock.

- scripts.js: this is file of pattern layout, it manages state of  side navigator in dashboard

- login.js : it will wait until users press login button, then it will read email and password of users, make a JSON string and send to IoT Core, my sever will check and send a response, if information is correct, users will be directory to users page, or not, users will stand at login page with an error announcement.

- register.js: I require 3 input : email, password, confirm password, it will send API of that parameters to sever to process, if information is correct, secer will return 1 response to redirect client to login page, or not, users will stand at register page with an error announcement.

- updateID.js: it will read information with 2 parameters like login, but it will send this pakage to router ‘/user/update’.

- updatePass.js: it reads 3 input parameters : email, password and newpassword. The package will sent to ‘/user/update’.

- delete.js: it will read information with 2 parameters like login, but it will send package to router ‘/user/delete’, it support

- verify_email.js: it will send email of users who forget their password to router ‘/forgetpass’, this file support forgetpass page.

- verify_number.js: it will send email of users and number they receive in email to ‘/forgetpass/verify’, this file support verify page.

- datatables.js: it will send request to get data for tables feature, which pages add this file can get data continuously.

- chart-line.js: it includes function to init line-chart, function to get data continuously and function to clear data when full.

- chart-bar.js: it includes function to init bar-chart, function to get data continuously.

- chart-control.js: when users press “Choose”, this file will catch event “click” and  set variable “start” to 1, if this variable is 1, program will start to initialize and get data.

- configDev.js: after users fill all parameters for dev and press “add”/”remove” button, it will pack data and send to sever, and wait response .

- configEquipments.js: it wil read state of  switch button on card which allow users to disable or enable equipments, then send to sever.

- configEquipmentsMark.js: it will read names of equipments which you config, then send to sever.

- configEquipmentsMaskport.js: it read name of port which you config, one port represent one “block”, then send to sever.

- configEquipmentsPin.js: it read index number of pin on device which you config, then send to sever.

- configEquipmentsPort.js: it read index number of port which you config, every port has 1 unique number, then send to sever.

- configEquipmentsType.js: it wil read state of  switch button on card which allow users to choose type of sensor, then send to sever.

- configEquipmentsTable.js: it read name of device which control equipments, in the search box of equipment page, then pack and send this information to sever to check.

- configSensors.js: it wil read state of  switch button on card which allow users to disable or enable sensors, then send to sever.

- configSensorsMask.js: it will read names of sensors which you config, then send to sever.

- configSensorsType.js: it wil read state of  switch button on card which allow users to choose type of sensor, then send to sever.

- configSensorsTable.js: it read name of device which control sensors, in the search box of sensor page, then pack and send this information to sever to check.

- configBlock.js: after users fill all parameters for block and press “add”/”remove” button, it will pack data and send to sever, and wait response .

- configBlock_Digi.js: it wil read state of  switch button on card which allow users to turn on or turn off equipments, then send to sever.

- configBlock_Vol.js: it wil read value of  slider on card which allow users to adjust value of DAC or PWM, then send to sever.

- configBlockTables.js: it read name of block in the search box of block page, then pack and send this information to sever to check.

Fuction pattern to send request to sever, example for sending request with POST method to login router:

---------------------------------------------------------------------------------------------

let response = await fetch('https://iot-server-365.herokuapp.com/login',{

method: 'post',

mode: 'cors',

headers:{

//'Accept': 'application/json, text/plain, */*',

'Content-Type': 'application/json'

},

body:JSON.stringify(frame)

});

--------------------------------------------------------------------------------------------

Fuction pattern to send request to sever, example for sending request with GET method to getdata router:

--------------------------------------------------------------------------------------------

let response = await fetch('https://iot-server-365.herokuapp.com/user/display/getdata', {

method: 'get',

mode: 'cors',

headers: {

'Accept': 'application/json, text/plain, */*',

'Content-Type': 'application/json',

id:dev

}

});

- documents

It includes old files, when I build the system, some errors occured and some GUI were obsolete . But it is the premise for me to build a good system now.

Some method I can reuse or improve in the future.

- node_modules:

These are the modules of nodejs, based on JavaScript, TypeScript and other languages. It includes package I added in the package.json file, too.

In conclusion, with inheritance ideal from MVC structure, and refer some direction of developer on github, I have complete my IoT core successfully.

# Chapter 3:

# MongoDB

3.1 Intruduction

3.1.1 Database

Concept of database (Oracle wesite https://www.oracle.com/database/what-is-database.https://www.oracle.com/database/what-is-database.html#:~:text=Products%20A%E2%80%93Z-,Database,electronically%20in%20a%20computer%20system.&text=The%20data%20can%20then%20be,for%20writing%20and%20querying%20data..

)

“A database is an organized collection of structured information, or data, typically stored electronically in a computer system. A database is usually controlled by a database management system (DBMS). Together, the data and the DBMS, along with the applications that are associated with them, are referred to as a database system, often shortened to just database”.

My system is service muilti-user, it will have problem if I use file system to save data of users. Other way, sever shouldn’t becom a storage, I need 1 storage area which is allow me to save data with a easy structure , but it can be access easily.

3.1.2 Databse management system

https://www.techopedia.com/definition/24361/database-management-systems-dbms

“A database management system (DBMS) is a software package designed to define, manipulate, retrieve and manage data in a database. A DBMS generally manipulates the data itself, the data format, field names, record structure and file structure. It also defines rules to validate and manipulate this data”.

https://www.guru99.com/sql-vs-nosql.html

3.1.3. SQL

“Structured Query language (SQL) pronounced as "S-Q-L" or sometimes as "See-Quel" is the standard language for dealing with Relational Databases. A relational database defines relationships in the form of tables.”

3.1.4. NoSQL database

https://www.mongodb.com/nosql-explained

“NoSQL databases (aka "not only SQL") are non tabular, and store data differently than relational tables. NoSQL databases come in a variety of types based on their data model. The main types are document, key-value, wide-column, and graph. They provide flexible schemas and scale easily with large amounts of data and high user loads.”

3.2 MongoDB

https://en.wikipedia.org/wiki/MongoDB

“MongoDB is a cross-platform document-oriented database program. Classified as a NoSQL database program, MongoDB uses JSON-like documents with optional schemas. MongoDB is developed by MongoDB Inc. and licensed under the Server Side Public License”.

MongoDB is supported on many platforms , it includes nodejs, it is easy to use for applications like my sever. MongoDB support a global cloud database service, it will storage data free : Up to 5GB storage and Shared RAM.

3.3 Mongoose

https://www.freecodecamp.org/news/introduction-to-mongoose-for-mongodb-d2a7aa593c57/

“Mongoose is an Object Data Modeling (ODM) library for MongoDB and Node.js. It manages relationships between data, provides schema validation, and is used to translate between objects in code and the representation of those objects in MongoDB”.

With mongoose, I can make a model for document easily, it supports many function with filters when fine, update, delete, create documents.

3.4 MongoDB Atlas

https://www.mongodb.com/cloud/atlas

“MongoDB Atlas is the global cloud database service for modern applications. Deploy fully managed MongoDB across AWS, Azure, or GCP. Best-in-class automation and proven practices guarantee availability, scalability, and compliance with the most demanding data security and privacy standards”.

A cloud database will allow my sever access from other cloud and hand to the database. This is my my batabase stored on MongoDB Atlas:

Chapter 4:

DEVICE

4.1.Introduce about ESP32 Microcontroller

https://datasheet.octopart.com/ESP32-D0WDQ6-Espressif-Systems-datasheet-138896080.pdf

“ESP32 is a single chip 2.4 GHz Wi-Fi and Bluetooth combo chip designed with TSMC ultra low power 40 nm technology. It is designed and optimized for the best power performance, RF performance, robustness, versatility, features and reliability, for a wide variety of applications, and different power profiles”.

CPU of ESP32 is Xtensa dual-core (or single-core) 32-bit LX6 microprocessor, it operates at 160 or 240 MHz and performing at up to 600 DMIPS, and supports Ultra low power (ULP) co-processor. In addition, it have memory is 512 KiB SRAM.

In my thesis, I choose esp32-wrover module and esp32 pico D4 kit for my thesis:

- Esp32 wrover module : it supports PSRAM make it more powerful.

https://docs.espressif.com/projects/esp-idf/en/latest/esp32/hw-reference/modules-and-boards.html

- Esp32 pico D4 kit : it use ESP-PICO-D4 is a System-in Package(SiP) module base on ESP32

Proe

4.2. Adapter for  ESP32-WROVER-V4.

Because ESP32-WROVER-V4 module needs a adapter to be programed and connect to sensors or equipments. I has make a adapter for it with support from KiCad sortware.

4.1.2.Schelmatic

ADC Input

Buck Converter 5v to 3v3

Boot button and reset button

DAC

GPIO

CAN

4.1.2. PCB  images.

PCB-top

PCB-bottom

4.1.3. Production.

4.2.Introduce ESP-IDF

It mean Espressif IoT Development Framwork. This framework support many API for peripherals, WI-Fi, Protocols,…

When I use ESP-IDF and Arduino platform, I have some compare : ESP-IDF is more flexible, easier for using and develop library, when esp32 use Wi-Fi, with arduino make devide hot and crash, othewise, ESP-IDF make esp32 operation normally.

In additionally, ESP-IDF integrate FreeRTOS, a real-time operating system, and we will have power from 2 core

4.3.Feature of device

The most important feature is HTTP protocol, because sever will response API through HTTP. In the first time, I think esp32 should be a sever, and then I see the power of nodejs , cloud resource, I decided esp32 must be client and it will use API of HTTP Client of ESP-IDF

Next is Wi-Fi, I do not want to pay for Ethernet wire , and Wi-Fi a wireless standard very common.

The Thirdly, with the “euipment” I use feature DAC,PWM ,Digital output to control it, with “sensor” I use ADC, I2C, Digital input to get data. Because my IoT core is not involved with device, developer can use other standard communicate like UART,SPI,… to get data or control somethings

4.4.Algorithm flowchart

With FreeRTOS , I can distribute feature follow task , esp32 will proccess every task in their period. The task which communicate with sever will have the highest property to ensure it will be process in time.

I give the Algorithm flowchart below :

Algorithm flowchart of devices ESP32

If any error happen, I will restart the device, because error can be a crash a missing a data, it may be do harm device, restart wil ensure ESP32 has initialized all parameters.

Chapter 5:

USER

5.1 Key-word in my system

Device : index of esp32 which you choose ( 0 < device < 232-1)

sensor : which give data of enviroment ( orther microprocessing, sensor DHT21,...)

euipment : which you can control througth send command to  server ( didital , non-digital)

ID : Identify of unique User , User can have many device but only 1 ID, remember

Type Of devide : I divide into 2 type ( can be expanse in the future ) , basic is :  0 ( to get command from sever and do it )  and 1 ( to send data to server ). It is GET and POST in HTTP. Device type 0 manage Equipment ( receive command),device type 1 manage Sensor ( read data).

ConFig : it help you announce your device and add it to you profile

5.2 Pages of webap

5.2.1. Register Page.

If you want to use this application, you have to register an account. Email used for registering is gmail, if you fail, you will receive a notification in this page, or if you are successful, your browser will be redirected to Login Page and you have been an user. Then, users will receive ID in their email, they use this ID to config for devices.

5.2.2 Longin Page.

Only users have permission to sign in their account, anyone tries accessing to path ‘/user’ or sub-path of it will be redirected to Login Page. Every time, users sign in successfully, your browser will be redirected to User Page.In addition, if users forget their password, they can click “Forgot password?” to reset password.

5.2.3. Help Page.

In this page, users need to fill their email and then you will received 1 number. This number is used for verifying your email.

5.2.4. Verify Page.

It is very easy, you just fill email again and the number you received in email. If everything is correct, users will receive second email, this email contain a new password. Users should update password after log in with new password for safety of their information.

5.2.5 User Page.

With pattern dashboard, I can fix and add my options. When press the letter “Setting” on the left corner of top page, a list menu will appear. I will summarize in one flowchart.

The dashboard will display data of the first device, which control sensors, so that you can see detail of charts, data table, you need to go their pages.

I supoort 4 card, with 3 cards are used, 1 card may be used on the future, users can fine it on “Setting/GUI/Authentication”.

I turn off the data in the User Page, users can turn on 1 or more on 1 chart. I just support 6 data points on every chart, to see more, users need to go to Display Page.

Data table just show the lastest data of first device. To have more feature, users have to go to Data Table Page.

5.2.6 Display Page

This page include 2 chart like User Page, but it allow users to choose devices, and then press “Choose” button, charts will be update continuously

5.2.7 Data Table Page

In this page, I support to clone data with 2 options:

- Filling amount of data and press green button, users will get the lastest 1500 data.

- Selecting period time with resolution is 1 minute, then press blue buttons, users will get all data which sever can send.

5.2.8. Update Page

In this page, I support updating for email and ID. With 2 card for 2 purposes, users can update these parameter individually.

5.2.9. Delete Page.

In this page, users can delete your account, they should consider carefully because it is a 1-way street.

5.2.10 Config Page

This is an important page, because it will allow users add/remove devices which they have configed. In other way, users may increase or decrease amount of equipments/sensors of devices, it like you are updaing devices.

5.2.11. Equipment Page

I will show a record of devices and index of device in every element. I support 1 search bar, if users have many devices, it may be helpful to fine device with name.

5.2.12. Equipi Page

Each of devices has 1 page like below to users can config parameters. I make 7 card with 7 parameters, users needn’t to config all of them.

- This card allow users disable or enable equipments. After 1 equipment has been disabled, users can not change state of equipment.

- I make 2 type of equipment, digital and not-digotal. With digital type, it is control on/off with value of equipments is set to 1 or 0. With non-digital type, I just support users for controling PWM and DAC with value from 0 to 255.

- I allow users to change name of equipment, it is useful because users need name to distinguish each other

- Port is used to gather pins into block. Every ports should have a name, but you needn’t change here, it can change through creating block.

- Ports will have unique index, it help sever process easily.

- Pin is index of pin on device, pins have to in 1 device.

5.2.13. Sensor Page

It is like Equipment Page with 1 search bar and 1 record.

5.2.14. Sensi Page

Each of devices has 1 page like below to users can config parameters. I make 3 card with 3 parameters.

- This card allow users disable or enable sensors. After 1 sensor has been disabled, users can see it in list of sensors which is display on chart

- I make 2 kind of charts : line-chart and bar-chart.

- I allow users to change name of sensors, it will help users fine sensors they need easily and turn it on.

5.2.15. GUI Page

The index of block and it’s mask will be update with index of port and port’s mask in device. Block is assigned to 1 device everytime config.

5.2.16 Block Page

It is like Equipment Page with 1 search bar to fine block and 1 record to show name and index of block..

5.2.17. Control Block Page

I create 2 card, one for digital type and one for non-digital type, digital card include switch buttons, non-digital card include slider bars.

Equipments which are disable will have  apple icon or if it is enabled, it will have androi icon.

Chapter 6:

SUMMARY

Bibliography

- Nodejs 12.x.x document. Retrieved from https://nodejs.org/dist/latest-      v12.x/docs/api/.

- NPM 6.x.x document. Retrieved from https://docs.npmjs.com/.

- StartBootstrap-sb-admin. Retrieved from github https://github.com/StartBootstrap/  startbootstrap-sb-admin.

- ESP-IDF 4.1 document with API Reference. Retrieved from https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-      reference/index.html.

- ESP-IDF 4.1. Retrieved from github  https://github.com/espressif/esp-idf .

- Mongoose 5.xx document. Retrieved from https://mongoosejs.com/docs/documents.html.

- Mongodb 4.x . Retrieved from https://docs.mongodb.com/manual/core/document/.